ANTENNA TEST REPORT

Manufacturer:

Model：

Type：External Reader Antenna

A.  Specification

| Electrical Specifications |  |
| --- | --- |
| Frequency | 902-928MHz |
| Gain | 9dBi |
| Beam width | Hor：70° Ver：70° |
| Polarization | Circular |
| VSWR | ≤1.4 |
| F/B Ratio | ≥20 |
| Impedance | 50Ω |
| Maximum power | 50W |
| Lightning protection | DC Ground |
| Connector | N-Female |
| Mechanical Specifications |  |
| Dimension | 258*258*42mm |
| Net weight | 0.95kg |
| Packing size | 40.5*30.5*10.5cm |
| Gross weight | 1.95kg |
| Radome material | ABS |
| Radome colour | White |
| Pole Diameter | Φ40-60mm |
| Rated wind velocity | 60m/s |
| Operating temperature | -40-70℃ |

B．Engineering Drawings

C.  VSWR

- Azimuth & gain at different frequency points.

- Frequency point：902MHz

2. Frequency point：  915MHz。

3. Frequency point：928MHz